# Experiment 1.2

Student Name: Sahil Kaundal				UID: 21BCS8197

Branch: BE CSE (Lateral Entry)                             Section/Group: 807/B

Semester: 4th						Date of Performance: 08/03/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# 1. Aim/Overview of the practical:

Addition of two 16-bits numbers.

# 2. Task to be done/ Which logistics used:

To add a two 16-bit numbers in 8085 microprocessor.

3. Example:

Input data of 1st no.→

Memory Address→

Memory Address→

| 66 | 45 |
| --- | --- |
| 2051 | 2050 |

Input data of 2nd no.→

Memory Address→

Memory Address→

| 33 | 22 |
| --- | --- |
| 2053 | 2052 |

Carry

Output data→

Memory Address→

Memory Address→

↓

| 00 | 99 | 67 |
| --- | --- | --- |
| 3052 | 3051 | 3050 |

- Addition of 16 bit numbers using 8 bit operation – It is a lengthy method and requires more memory as compared to 16 bit operation.

4. Algorithms:

- Load the lower part of first number in B register

- Load the lower part of second number in A (accumulator)

- Add both the numbers and store

- Load the higher part of first number in B register

- Load the higher part of second number in A (accumulator)

- Add both the numbers with carry from the lower bytes (if any) and store at the next location

5. Programs:

| Memory Address | Mnemonics | Comment |
| --- | --- | --- |
| 2000 | LDA 2050 | A<-2050 |
| 2003 | MOV B, A | B<-A |
| 2004 | LDA 2052 | A<-2052 |
| 2007 | ADD B | A<-A+B |
| 2008 | STA 3050 | A->3050 |
| 200B | LDA 2051 | A<-2051 |
| 200E | MOV B, A | B<-A |
| 200F | LDA 2053 | A<-2053 |
| 2012 | ADC B | A<-A+B+CY |
| 2013 | STA 3051 | A->3051 |
| 2016 | HLT | Stops execution |

6. Explanation:

- LDA 2050 moves the contents of 2050 memory in A accumulator.

- MOV B, A stores the value of A into B register

- LDA 2052 stores the value at 2052 in A

- ADD B add the contents of B and A and store in A

- STA 3050 stores the result in memory location 3050

- LDA 2051  stores the value at 2051 in A

- MOV B, A stores the value of A into B register

- LDA 2053  stores the value at 2053 in A

- ADC B add the contents of B, A and carry from the lower bit addition and store in A

- STA 3051 stores the result in memory location 3051

- HLT stops execution.

- Addition of 16 bit numbers using 16 bit operation – It is a very short method and less memory is also required as compared to 8 bit operation.

7. Algorithm:

- Load both the lower and the higher bits of first number at once

- Copy the first number to another register pair

- Load both the lower and the higher bits of second number at once

- Add both the register pairs and store the result in a memory location.

8. Program:

LHLD 1000

XCHG

LHLD 1002

MVI C,00

DAD D

JNC LABEL

INR C

MOV A,C

STA 1006

LABEL:	   SHLD 1004

HLT

# ORG 1000

# DB 89,78,55,60

| Memory Address | Mnemonics | Comments |
| --- | --- | --- |
| 2000 | LHLD 2050 | H-L<-2050 |
| 2003 | XCHG | D<->H & E<->L |
| 2004 | LHLD 2052 | H-L<-2052 |
| 2007 | DAD D | H<-H+D & L<-L+E |
| 2008 | SHLD 3050 | A->3050 |
| 200B | HLT | Stops execution |

9. Explanation:

- LHLD 2050 loads the value at 2050 in L register and that in 2051 in H register (first number)

- XCHG copies the content of H to D register and L to S register

- LHLD 2052 loads the value at 2052 in L register and that in 2053 in H register (second number)

- DAD D adds the value of H with D and L with E and stores the result in H and L

- SHLD 3050 stores the result at memory location 3050

- HLT stops execution

# 10. Result/Output/Writing Summary:

I have successfully completed this experiment.

Learning outcomes (What I have learnt):

- Addition of two 16-bits numbers.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |