# Experiment 1.3

Student Name: Sahil Kaundal				UID: 21BCS8197

Branch: BE CSE (Lateral Entry)                             Section/Group: 807/B

Semester: 4th						Date of Performance: 08/03/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

# (a)

# 1. Aim/Overview of the practical:

Subtraction of two 8 bit numbers along with considering borrow.

- Task to be done:

Subtraction of two 8 bit numbers along with considering borrow.

- Apparatus/Simulator used (For applied/experimental sciences/materials based labs):

    - Jubin Application

    - 8085 Simulator

    - JDK

- Algorithm/Flowchart (For programming based labs):

  - Load the H-L pair with the address of first memory location.

  - Move the content of H-L to accumulator.

  - Increment H-L pair to next memory location.

  - Load the register B with the memory location of second data.

  - Initialize register C with 00H. this will store the borrow (if any).

  - Subtract the content of accumulator with the content of register B and the result will be stored in accumulator automatically.

  - If carry flag is set then increment register C.

  - Increment H-L pair.

  - Move result from accumulator to memory location 3002H.

  - Increment H-L pair.

  - Move borrow from register C to memory location 3003H.

- Description/ Code:

LXI H,7501

MOV A,M

INX H

SUB M

MOV M,A

HLT

# ORG C050

# DB 90H,65H

- Result/Output/Writing Summary:

(b)

  - Aim/Overview of the practical:

Subtraction of two 16 bit numbers along with considering borrow

  - Task to be done:

Subtraction of two 16 bit numbers along with considering borrow

  - Apparatus/Simulator used :

    - Jubin Application

    - 8085 Simulator

    - JDK

  - Algorithm/Flowchart:

      - Load 0000H into CX register (for borrow)

      - Load the data into AX(accumulator) from memory 3000

      - Load the data into BX register from memory 3002

      - Subtract BX with Accumulator AX

      - Jump if no borrow

      - Increment CX by 1

      - Move data from AX(accumulator) to memory 3004

      - Move data from CX register to memory 3006

      - Stop

# Description/ Code:

LHLD 7501

XCHG

LHLD 7503

MOV A,E

SUB L

MOV L,A

MOV A,D

SBB H

MOV H,A

SHLD 7505

HLT

# ORG 7501

# DB 90,70,10,30

  - Result/Output/Writing Summary:

Learning outcomes (What I have learnt):

- Working of microprocessors.

- Learn how to do mathematical operations in microprocessors.

- Learn about 8085 simulator.

- Subtraction of two 8 bit numbers

- Learn about the different instructions that are needed to be given to the memory to perform some tasks.

- Subtraction of two 16 bit numbers

Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |