# Experiment 7

Student Name: Sahil Kaundal				UID: 21BCS8197

Branch: BE CSE (Lateral Entry)                             Section/Group: 807/B

Semester: 4th						Date of Performance: 14/04/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

- Aim/Overview of the practical:

# Shifting of 16-bit numbers.

# 2. Task to be done/ Which logistics used:

a) Shift a 16-bit number to left by 1-bit

b) Shift a 16-bit number to left by 2-bits.

3. Apparatus / Simulator Used:

1. Jubin Application

2. 8085 Simulator

3. JDK

4. Algorithms/Flowcharts:

Shift a 16 -bit number to left by 1-bit:

1. Load H – L pair with address 3000H.

2. DAD is used to shift the 16-bit number to left by 1-bit.

3. Store HL pair using direct addressing in memory location.

4. Terminate the program.

Shift a 16 -bit number to left by 2-bits:

1. Load H – L pair with address 3000H.

2. DAD is used to shift the 16-bit number to left by 2-bits.

3. Store HL pair using direct addressing in memory location.

4. Terminate the program.

5. Programs:

Shift a 16 -bit number to left by 1-bit:

# BEGIN 0000H

LHLD 3000H

DAD H

SHLD 3001H

HLT

# ORG 3000

# DB 96H,75H

Shift a 16 -bit number to left by 2-bits:

# BEGIN 0000H

LHLD 3000

DAD H

DAD H

SHLD 3001

HLT

# ORG 3000

# DB 96H,75H

# 6. Result/Output/Writing Summary:

a) Shift a 16 -bit number to left by 1-bit:

b) Shift a 16-bit number to left by 2-bits:

Learning outcomes (What I have learnt):

1. Working of microprocessors.

2. Learn how to shift data in microprocessors.

3. Learn about 8085 simulator.

4. Operations of 16-bit numbers.

5. Learn about the different instructions that are needed to be given to the memory to perform some tasks.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |