# Final Practical Exam

Student Name: Sahil Kaundal				UID: 21BCS8197

Branch: BE CSE (Lateral Entry)                             Section/Group: 807/B

Semester: 4th						Date of Performance: 20/05/2022

Subject Name: MPI Lab	                               	Subject Code: 22E-20CSP-253

- Aim/Overview of the practical:

# Write a program to find the 2’s complement of 16-bit number stored in memory locations 3000H and 3001H.

# 2. Task to be done/ Which logistics used:

# Write a program to find the 2’s complement of 16-bit number stored in memory locations 3000H and 3001H.

3. Apparatus / Simulator Used:

1. Jubin Application

2. 8085 Simulator

3. JDK

4. Algorithms/Flowcharts:

- A program to find the 2’s complement of 16-bit number stored in memory locations 3000H and 3001H.

- Load a 16-bit number from memory 3000 into a register pair (H-L)

- Move content of register L to accumulator

- Complement content of accumulator

- Move content of accumulator to register L

- Move content of register H to accumulator

- Complement content of accumulator

- Move content of accumulator to register H

- Store content of register pair in memory 3002 (1’s complement)

- Increment content of register pair by 1

- Store content of register pair in memory 3004 (2’s complement)

- Stop

5. Programs:

- A program to find the 2’s complement of 16-bit number stored in memory locations 3000H and 3001H.

# BEGIN 0000H

LXI H,3000

MVI B,00

MOV A,M

CMA

ADI 01

STA 3002

JNC GO

INR B

GO:	   INX H

MOV A,M

CMA

STA 3003

HLT

# ORG 3000

# DB 12H,05H

# 6. Result/Output/Writing Summary:

- A program to find the 2’s complement of 16-bit number stored in memory locations 3000Hand 3001H.

# I Have Successfully Done This Practical.

# Evaluation Grid (To be created as per the SOP and Assessment guidelines by the faculty):

| Sr. No. | Parameters | Marks Obtained | Maximum Marks |
| --- | --- | --- | --- |
| 1. |  |  |  |
| 2. |  |  |  |
| 3. |  |  |  |